Air-Breathing Engine Design Notes

What causes a compressors operating line to shift right towards choking?

- Reduced downstream pressure

  - Opening nozzle area, lower turbine PR, reduced combustor pressure losses.

- Lower inlet temperature

  - Colder inlet air is denser. For same volumetric flow, more mass enters inlet.

- Engine throttle changes

  - Transient operation may push line to choke

Left shift = less flow / more back pressure / surge risk

Right shift = more flow / less back pressure / choke risk

What causes Lean Blowout (LBO) in an engine?

- Fuel-Air mixture too lean

  - Less fuel energy in air

  - Flame temperature drops

  - Reaction rates slow down

- Residence Time Decreases (Fuel/Air not in high temperature area long enough)

- Flame Stabilization Breakdown

- Excessive Heat Loss

- Poor Atomization or Mixing of Fuel

- LBO is approached during idle, high altitude relight, rapid deceleration, and low-power operation.